# FFT-Based Water Leak Detection System - Line by Line Explanation

1. Library Include
#include <arduinoFFT.h>
This library enables Fast Fourier Transform (FFT) operations on Arduino. FFT converts time-domain signals into frequency-domain signals.

2. Pin Definitions
FLOW_PIN = 2 → Flow sensor input
RELAY_PIN = 8 → Controls solenoid valve
BUZZER_PIN = 7 → Alarm indicator
LED_PIN = 6 → Visual indicator

3. FFT Parameters
SAMPLES = 64 → Number of samples (must be power of 2)
SAMPLING_INTERVAL = 200 ms → Sampling every 200ms
Sampling frequency Fs = 1 / 0.2 = 5 Hz

4. FFT Object Creation
ArduinoFFT<double> FFT;
Creates FFT object using double precision.

5. Global Variables
volatile unsigned int pulseCount = 0;
Counts pulses from flow sensor (volatile because used in interrupt).

double vReal[64], vImag[64];
Stores real and imaginary parts for FFT.

unsigned long lastSampleTime = 0;
int sampleIndex = 0;
Tracks sampling timing and array index.

6. Interrupt Function
void pulseCounter() { pulseCount++; }
Triggered on each rising pulse from flow sensor.

7. Setup Function
Serial.begin(9600); → Enables serial monitor
Pin modes set as INPUT/OUTPUT.
Relay HIGH → Valve open
Buzzer & LED LOW → OFF
attachInterrupt(...) → Starts pulse counting automatically.

8. Sampling Logic in loop()
Every 200ms:
- Detach interrupt
- Calculate flowRate = pulseCount / 7.5 (L/min)
- Reset pulseCount
- Reattach interrupt
- Store flowRate into vReal array
- When 64 samples collected → Call analyzeFFT()

9. FFT Analysis
Step 1: Compute Mean (DC component)
Step 2: Remove DC offset from signal
Step 3: Apply Hamming Window (reduces spectral leakage)
Step 4: Compute FFT
Step 5: Convert complex values to magnitude

10. DC Ratio Calculation
dcEnergy = vReal[0] → DC component
totalEnergy = Sum of frequency magnitudes
dcRatio = dcEnergy / totalEnergy

11. Leak Detection Logic
If meanFlow > 0.2 L/min AND dcRatio > 0.7:
→ Leak Detected
→ Close valve (Relay LOW)
→ Turn ON buzzer and LED

Else:
→ System normal
→ Valve open
→ Alarm OFF

Concept:
Normal water use → Fluctuating flow → Low DC ratio
Leak → Constant steady flow → High DC ratio

This system detects steady continuous water flow patterns using frequency-domain analysis instead of simple threshold detection.