uno
#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <SoftwareSerial.h>

#include <arduinoFFT.h>

// ---------------- LCD & Bluetooth ----------------

LiquidCrystal_I2C lcd(0x27, 16, 2);

SoftwareSerial BT(8, 7);   // RX, TX

// ---------------- Pins ----------------

const int trig = 9;        // Tank ultrasonic

const int echo = 10;

const int irLeakPin = 12;  // IR leak sensor digital output

const int redLED = 5;

const int buzzer = 4;

const int flowPin = 2;     // Flow sensor (interrupt)

// ---------------- Tank Calibration ----------------

float emptyDistance = 9.52;  // cm when tank empty

float fullDistance  = 2.47;  // cm when tank full

// ---------------- Flow Variables ----------------

volatile unsigned long pulseCount = 0;

unsigned long lastFlowTime = 0;

float flowRate = 0;   // L/min

// ---------------- FFT Variables ----------------

#define SAMPLES 64

#define SAMPLING_INTERVAL 200 // ms

ArduinoFFT<double> FFT;

double vReal[SAMPLES];

double vImag[SAMPLES];

unsigned long lastSampleTime = 0;

int sampleIndex = 0;

// ---------------- Flow Limits ----------------

float minFlow = 1.0;

unsigned long lowFlowStartTime = 0;

bool lowFlowLeakActive = false;

// ---------------- Leak Detection ----------------

bool leakDetected = false;

// ---------------- Interrupt ----------------

void pulseCounter() {

pulseCount++;

}

// ---------------- Setup ----------------

void setup() {

pinMode(trig, OUTPUT);

pinMode(echo, INPUT);

pinMode(irLeakPin, INPUT);  // IR sensor input

pinMode(redLED, OUTPUT);

pinMode(buzzer, OUTPUT);

pinMode(flowPin, INPUT);

digitalWrite(redLED, LOW);

digitalWrite(buzzer, LOW);

attachInterrupt(digitalPinToInterrupt(flowPin), pulseCounter, RISING);

lcd.init();

lcd.backlight();

BT.begin(9600);

Serial.begin(9600);

}

// ---------------- Measure Distance ----------------

float measureDistance(int trigPin, int echoPin) {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH, 30000);

if (duration == 0) return -1;

return duration * 0.0343 / 2;

}

// ---------------- Loop ----------------

void loop() {

// -------- Ultrasonic Tank Level --------

float tankDistance = measureDistance(trig, echo);

float tankPercentage = 0;

if (tankDistance > 0) {

tankPercentage = (emptyDistance - tankDistance) / (emptyDistance - fullDistance) * 100.0;

tankPercentage = constrain(tankPercentage, 0, 100);

}

// -------- Flow Calculation (1 sec) --------

if (millis() - lastFlowTime >= 1000) {

detachInterrupt(digitalPinToInterrupt(flowPin));

flowRate = pulseCount / 7.5;  // YF-S201

pulseCount = 0;

attachInterrupt(digitalPinToInterrupt(flowPin), pulseCounter, RISING);

lastFlowTime = millis();

// Low flow timer

if (flowRate < minFlow && flowRate > 0) {

if (lowFlowStartTime == 0) lowFlowStartTime = millis();

else if (millis() - lowFlowStartTime >= 5000) lowFlowLeakActive = true;

} else {

lowFlowStartTime = 0;

lowFlowLeakActive = false;

}

}

// -------- FFT Sampling --------

if (millis() - lastSampleTime >= SAMPLING_INTERVAL) {

vReal[sampleIndex] = flowRate;

vImag[sampleIndex] = 0;

sampleIndex++;

lastSampleTime = millis();

if (sampleIndex >= SAMPLES) {

analyzeFFT();

sampleIndex = 0;

}

}

// -------- Leak Detection (IR Sensor) --------

// Assume: LOW = leak detected, HIGH = no leak (typical for IR water sensor)

leakDetected = (digitalRead(irLeakPin) == LOW);

// -------- Alert Logic --------

bool anyAlert = leakDetected || lowFlowLeakActive || (tankPercentage >= 100);

digitalWrite(redLED, anyAlert ? HIGH : LOW);

// -------- Buzzer & Bluetooth --------

if (leakDetected) {

digitalWrite(buzzer, HIGH);

BT.println("IR LEAK DETECTED!");

} else if (lowFlowLeakActive) {

digitalWrite(buzzer, HIGH);

BT.println("LOW FLOW LEAK DETECTED!");

} else if (tankPercentage >= 100) {

digitalWrite(buzzer, HIGH);

BT.println("TANK FULL ALERT!");

} else {

digitalWrite(buzzer, LOW);

}

// -------- LCD Display --------

lcd.clear();

lcd.setCursor(0,0);

lcd.print("Water Left:");

lcd.setCursor(0,1);

lcd.print(tankPercentage,0);

lcd.print("%");

delay(500);

}

// ---------------- FFT Analysis ----------------

void analyzeFFT() {

// ---- Remove DC Offset ----

double mean = 0;

for (int i = 0; i < SAMPLES; i++) mean += vReal[i];

mean /= SAMPLES;

for (int i = 0; i < SAMPLES; i++) vReal[i] -= mean;

// ---- Windowing & FFT ----

FFT.windowing(vReal, SAMPLES, FFT_WIN_TYP_HAMMING, FFT_FORWARD);

FFT.compute(vReal, vImag, SAMPLES, FFT_FORWARD);

FFT.complexToMagnitude(vReal, vImag, SAMPLES);

// ---- DC Ratio ----

double dcEnergy = vReal[0];

double totalEnergy = 0;

for (int i = 0; i < SAMPLES/2; i++) totalEnergy += vReal[i];

double dcRatio = 0;

if (totalEnergy != 0) dcRatio = dcEnergy / totalEnergy;

Serial.print("Mean Flow: "); Serial.print(mean);

Serial.print(" | DC Ratio: "); Serial.println(dcRatio);

// ---- Leak Detection via FFT ----

if (mean > 0.2 && dcRatio > 0.7) {

BT.println("FFT LEAK DETECTED!");

digitalWrite(redLED, HIGH);

digitalWrite(buzzer, HIGH);

}

}

nano

#include <Servo.h>

#include <SoftwareSerial.h>

Servo myServo;

SoftwareSerial bt(10, 11);   // RX, TX

const int statePin = 2;

const int trigPin = 4;

const int echoPin = 5;

String inputPassword = "";

String correctPassword = "1234";

bool previousState = LOW;

bool doorOpen = false;

void setup() {

myServo.attach(9);

myServo.write(0);

pinMode(statePin, INPUT);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

bt.begin(9600);

Serial.begin(9600);

}

long getDistance() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH);

long distance = duration * 0.034 / 2;

return distance;

}

void loop() {

bool currentState = digitalRead(statePin);

// Ask password once when connected

if (currentState == HIGH && previousState == LOW) {

bt.println("=== Bluetooth Door Lock ===");

bt.println("Enter Password:");

inputPassword = "";

}

previousState = currentState;

// Handle Password

if (currentState == HIGH && bt.available()) {

char c = bt.read();

if (c == '\r' || c == '\n') {

inputPassword.trim();

if (inputPassword.equals(correctPassword)) {

bt.println("Access Granted");

// Smooth open

for (int pos = 0; pos <= 180; pos++) {

myServo.write(pos);

delay(10);

}

doorOpen = true;

bt.println("Door Open - Waiting for object within 7 cm");

} else {

bt.println("Wrong Password");

}

inputPassword = "";

bt.println("Enter Password:");

}

else {

inputPassword += c;

}

}

// Ultrasonic Distance Monitoring

if (doorOpen) {

long distance = getDistance();

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

if (distance > 0 && distance <= 7) {

bt.println("Object within 7 cm - Closing Door");

// Smooth close

for (int pos = 180; pos >= 0; pos--) {

myServo.write(pos);

delay(10);

}

doorOpen = false;

}

delay(200);  // small stability delay

}

}